Aurisys Development Tutorial

User Manual

Customer Support

MT6797

CS6797-XXX-UMD-V1.0EN

V1.0

2016-08-29

Internal

Doc No:

Version:

Release date:

Classification:

© 2016 MediaTek Inc.

This document contains information that is proprietary to MediaTek Inc.

Unauthorized reproduction or disclosure of this information in whole or in part is strictly prohibited.

Specifications are subject to change without notice.

Keywords

User Manual

MediaTek Inc.

Postal address

No. 1, Dusing 1st Rd. , Hsinchu Science Park, Hsinchu City, Taiwan 30078

MTK support office address

No. 1, Dusing 1st Rd. , Hsinchu Science Park, Hsinchu City, Taiwan 30078

Internet

http://www.mediatek.com/

Document Revision History

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| V0.8 | 2015/12/31 | Doug Wang | First Release |
| V0.9 | 2016/1/21 | Doug Wang | Add code release partand modify dsp build command |
| V0.9a | 2016/3/28 | Doug Wang | Add Software patch table |
| V1.0 | 2016/08/09 | Doug Wang | Add Library integration guidance in Application Processor; Record & VOIP Applications; XML Descriptor |

Table of Contents

Document Revision History	3

Table of Contents	4

Lists of Tables	7

Lists of Figures	8

1	Overview	9

1.1	Abstract	9

1.2	Aurisys Aims	9

1.3	Aurisys Structure	10

1.4	DSP Capability	11

1.4.1	DSP Basics	11

1.4.2	DSP Feature Packages	11

2	Getting Start with Aurisys	13

2.1	Comments	13

2.2	File Structure	13

2.2.1	DSP	13

2.2.2	Application Processor	14

2.3	Step by Step Integration Guide	14

2.3.1	Integrate Software IP in DSP	14

2.3.2	Integrate Software IP in Application Processor	17

3	Software Development Guidance	20

3.1	Software Interface Overview	20

3.2	Sound Processing Task Integration Guidance	20

3.2.1	ARSI-PROC	20

3.2.2	ARSI-PROC Functions	20

3.2.3	Example	22

3.3	Parameter Tuning Guidance	23

3.3.1	ARSI-PAR Usage	23

3.3.2	Communication based on Android Debug Bridge(adb)	24

3.3.3	ARSI-PAR Functions	25

3.3.4	Example	26

4	XML Descriptor for Aurisys	28

4.1.1	Example	28

4.1.2	Elements	29

4.1.3	Deployment	33

5	Debugging	34

5.1	Logging	34

5.1.1	Inside Software IP	34

6	Applications	35

6.1	Voice Call	35

6.1.1	Data Path	35

6.1.2	Basic Information	36

6.1.3	Limitations	37

6.1.4	Configurations	37

6.1.5	Software Frameworks	38

6.2	Playback	39

6.2.1	Data Path	39

6.2.2	Configurations	40

6.2.3	Behavior	40

6.2.4	Functional Blocks	40

6.3	Sound Trigger	41

6.4	SmartPA	41

6.5	Record & VOIP	42

6.5.1	Data Path	42

6.5.2	Configurations	42

6.5.3	Application Processor Only	43

6.5.4	Class Diagram	43

6.5.5	Device Information	43

7	Appendix: Platform and Development Environments	46

7.1	Scenario Based DVFS	46

7.1.1	Implementation	46

7.2	DSP Binary (Build Command and Download)	47

7.2.1	Configuration	47

7.2.2	Build Command	48

7.2.3	Built Binary	48

7.3	Code Release	49

7.4	Aurisys Framework Class Diagram	50

7.5	Software Patch Note	50

7.6	FAQ	51

Lists of Tables

Table 3-1. Adb command table	24

Table 3-2.Command argument table	24

Lists of Figures

Figure 1-1. Concept of Aurisys	9

Figure 1-2. Aurisys Structure	10

Figure 1-3. Possible DSP Software Packages of MTK Built-in Features	12

Figure 3-1. Parameter Path	23

Figure 4-1. Relationship between Scene Handler, Scenario Descriptor, and Library Descriptor	33

Figure 6-1. Hand-Held Mode Voice Call Data Path	35

Figure 6-2. Hand-Free Mode Voice Call Data Path	36

Figure 6-3. Software Frameworks of Voice Call	38

Figure 6-4. Offload Audio Playback	39

Figure 6-5. Functional Blocks of Offload Playback	41

Figure 6-6. Record & VOIP	42

Figure 6-7. Aurisys Class Diagram for Record & VOIP	43

Figure 7-1. MT6797 DSP Dynamic Voltage Table	46

Figure 7-2. Update DSP Binary by Flashtool	49

Figure 7-3. Aurisys Framework Class Diagram	50

# Overview

## Abstract

The requirement for sound quality enhancements that add listening pleasure has been increasing. To enrich the listening experience of using mobile devices, there is an increasing number of sound processing solutions provided by vendors.

Aurisys is introduced to facilitate the sound processing solution development and use of MTK platforms. Aurisys is a framework constructed upon Android Audio Framework. It includes standardized interfaces for sound processing and tuning, integrated DSP sound subsystem, and software debug interface. This concept of Aurisys will be briefly described in this document.

## Aurisys Aims

As the following figure depicts, our target  is to make the replacement/integration of vendor IPs as convenient as possible. We defined a standardized interface to which is platform-independent, so that the software can be re-used. By following the user interface standards we create a more consistent experience for the developers.

Figure 1-1. Concept of Aurisys

## Aurisys Structure

Figure 1-2. Aurisys Structure

As depicted in above figure, the Aurisys structure contains Scene Handler, Library Manager, integrated DSP framework, Modem/Audio HW subsystems, and standardized software interfaces:

- Aurisys Scene Handler (ARSH)

The Aurisys Scene Handler is created as the middleware between the audio system and the sound processing IPs. It  is created scene by scene. For example, the Aurisys Playback Handler is served for playback effects in the scene of playback. It provides interfaces to call the corresponding IPs and manages the IPs.

- Aurisys Library Manager (ARLM)

The Aurisys Library Manager maintains the library information of each Aurisys Scene Handler. It contains the list and status of sound IPs.

- Aurisys Software Interface (ARSI)

ARSI is the abbreviation of Aurisys Software Interface. It is provided for interfacing with sound enhancing tasks. To process sounds, a unified interface which is portable between MTK platforms is provided. The interface to parse and transmit parameters between PC tool, APMCU, and DSP is also included.  The interface is designed for ease of use while being general enough so that new algorithms can be easily added to the existing frameworks. The difference between ARSI and ARSH is that ARSI provides interface between ARSH and sound IPs; while ARSH provides interface for Android Audio HAL to request for processing sounds.

- Modem

The modem means the modem IC. The MTK internal chipset contains a DSP to process voice enhancement and voice codec. In Aurisys structure, we disable the voice enhancement, but reserve codec in modem IC so that we can just add algorithms in the integrated open DSP in the Application side.

- Audio HW

It refers to the interface between processors and HW devices to transfer sound data. The devices include speakers, microphones, earphones, BT devices, USB devices, etc.

## DSP Capability

### DSP Basics

The basic information about DSP is summarized as follows:

- Integrated open DSP support. The embedded processor is ARM Cortex™-M4 processor

  - Floating-Point-Unit supported

  - The internal SRAM of DSP is 512 KB

  - Maximum clock rate reaches 354 MHz

  - No cache support

- Programmable DMA (burst of 4 by 16-bit) for DSP supported to transfer data to/from external memory

  - Not suggest to access external memory by the processor directly, it may cause unexpected problems. Please use DMA for large quantity of data transfer and just use IPI (inter-processor-interrupt) for the  transfers of small data volumes.

  - The DSP can access audio registers, internal memory of audio hardware, and voice data from the modem side during a voice call.

  - We’ve provided an API to trigger DMA, please use polling mode, not to use interrupt mode in MT6797

- FreeRTOS has been adopted as the supported operating

  - Communicate with Application processor by IPI (inter-processor-interrupt); There are 48 bytes data can be transferred by share memory by one IPI.

  - Dynamic loading of code is not supported

  - Clock rate can be switched between 110/224/354 MHz according to requirements of applications

### DSP Feature Packages

Since we don’t support cache or dynamic loading of code, the feature sets supported in DSP may be limited. In MT6797, we provide the following built-in features: sensor hub (in MT6797, we use single DSP for both sensor hub and audio features), MP3 decoder, sampling rate converter, and sound trigger. Besides, the sound enhancement for voice calls is driver ready and properly maintained. Customers can add sound enhancement library of 3rd party easily.

However, the customer should select what they really want in CortexM4 since the memory size is not enough for all features. For example, the sensor feature and MP3 decoder have an application processor version. Thus, if the customer wants to add a vendor’s algorithm for voice call, they can select the application processor version of sensor/MP3 to release the memory of the DSP. Besides, even the 3rd party voice call enhancement is not adopted, we still have a MTK built-in version in modem side.

The following table shows possible feature packages of the DSP:

Figure 1-3. Possible DSP Software Packages of MTK Built-in Features

# Getting Start with Aurisys

## Comments

We annotate our code with Doxygen style comments which are used to provide documentation in-line. Before each function, we put a comment block giving at least:

- @brief: a brief description

- @param: describe all of the parameters to the function

- @return: return value

- After the return information, there is a detailed description of the function (optional).

Example:

/** ****************************************************************************** * @brief Query the size of the working buffer * * @param p_arsi_task_config the task configure * @param p_working_buf_size the working buffer size. * * @return lib_status_t ****************************************************************************** */

This function is used to get the working buffer size for a Software IP. p_arsi_task_config contains a particular  configuration for the algorithm; p_working_buf_size updated by the Software IP indicates the required memory size under the specified configuration. The return information indicates if there is any unexpected error occurs.

## File Structure

### DSP

The root folder of DSP is located at vendor/mediatek/proprietary/tinysys/freertos/source.

| Path | File | Description |
| --- | --- | --- |
| project/CM4_A/mt6797/platform | platform.mk | Project configurations (common) |
| project/CM4_A/mt6797/[project] | ProjectConfig.mk | Project configurations (specific project) |
|  | CompilerOption.mk | Project specific compile options |
| middleware/lib/aurisys | arsi_api.c / arsi_api.h | Aurisys Software Interface |
| drivers/CM4_A/mt6797/audio/src | audio_task_phone_call.c | Aurisys phone call driver |
|  | audio_task_factory.c | Audio task service |
|  | audio_messenger_ipi.c | Interrupt handler to handle message from AP |
|  | audio_irq.c | Interrupt handler of audio hw irq |
|  | audio.c | Audio init/de-init entry functions |

### Application Processor

| Path | File | description |
| --- | --- | --- |
| device/mediatek/[project] | ProjectConfig.mk | Project configurations of AP |
| /device/mediatek/[project]/aurisys_param | *.dat | Parameter file |
| /vendor/mediatek/proprietary/external/aurisys | *.so, *.xml | Parser library and Processing library (if in Application processor); XML Descriptor file |
| vendor/mediatek/proprietary/hardware/audio/common/V3 | AudioAurisysCommon.cpp/ AudioAurisysConfigParser.cpp/ AudioAurisysLibManager.cpp/ | Aurisys services |
|  | AudioAurisysXXXHandler.cpp | Aurisys scene handler |

## Step by Step Integration Guide

Before looking deep into Aurisys, we provide step-by-step guidance on how to integrate your program into Aurisys. It can be separated into two conditions: 1) Add or replace a SWIP in DSP; 2) Add or replace a SWIP in Application processor. We’ll take the integration of a voice call SWIP as an example of adding a SWIP in DSP. Note that the addition or replacement of a SWIP is based on an existing driver-ready scene of Aurisys.

### Integrate Software IP in DSP

#### Integration of a Voice Call SWIP (as example)

- Configurations

  - DSP Configurations

    - Common Configurations

The platform Makefile path: /project/CM4_A/[project]/platform/platform.mk

The CFG_AUDIO_SUPPORT is a compile option for audio framework, it will be enabled if there is at least one audio application is enabled.

    - Project Specific Configurations

The file “/project/CM4_A/[project]/[project]/ProjectConfig.mk” is used to enable or disable a feature for a specified project. CFG_MTK_AURISYS_PHONE_CALL_SUPPORT should be set as “yes” to enable voice call application in DSP. Besides, CompilerOption.mk is used for adding project specific compile options.

    - Heapsize Adjustment

The data memory can be managed by malloc functions. However, we need to define a appropriate heap size first. The default heap size for all projects is defined in the following file /project/CM4_A/[project]/platform/inc/FreeRTOSConfig.h. Then find the follow definition “#define configTOTAL_HEAP_SIZE ( ( size_t ) ( 120 * 1024 ) )”. We can also override the definition for a specific project in CompilerOption.mk mentioned above. Use the following definition to override the default heap size: CFLAGS += -DconfigTOTAL_HEAP_SIZE='( ( size_t ) ( 180 * 1024 ) )'.

  - AP Configurations

    - Projet Configurations

The file “device/mediatek/[project]/ProjectConfig.mk” is used to enable or disable a feature for a specified project of application processor. Set MTK_AURISYS_PHONE_CALL_SUPPORT as “yes”. Notice that there is an automatic compile dependency check between “PROJECT_CONFIG” of AP and “CFG_PROJECT_CONFIG” of DSP. They should be the same value.

    - Clock rate requirement

We don’t have automatic DVFS to detect current DSP loading and adjust the voltage automatically. What we do is to register a frequency requirement table for a specified application. When the application starts, a provided API should be called to notify the system. Besides, it also needs to call another API to notify the system when it is stopped. The system will adjust the voltage according to the sum of clock rate requirement for active applications. The value of clock rate requirement should be estimated precisely and then modify the corresponding valuethe following file: kernel-3.18 / drivers/misc/mediatek/scp/[project]/scp_helper.c.  Note that the feature ID: OPEN_DSP_FEATURE_ID is for voice call of Aurisys.

static scp_feature_table_t feature_table[] = { { …… …… { .feature = OPEN_DSP_FEATURE_ID, .freq = 270, .enable = 0, }, ……. ……. };

- Software IP Development

  - Delveop with ARSI

Follow ARSI and implement the functions defined in arsi_api.h. The detailed description of ARSI is in the next chapter.

  - DSP Binary

    - Build Command

Notice that the Android build environment is required. The build commands is listed as follows:

- $ . build/envsetup.sh

- $ lunch full_<PROJECT>-eng

- $ mosesq make -j24    (full Android Build)

- $ mosesq make tinysys-scp -j24  (build DSP only)

- $ vendor/mediatek/proprietary/tinysys/freertos/source/tools/build_ts.sh -j24  (fast DSP build)

    - Built DSP Binary Location

The built DSP binary is located in the following path:

out\target\product\[project]\obj\TINYSYS_OBJ\tinysys-scp_intermediates\freertos\source\CM4_A\tinysys-scp-CM4_A.bin

    - Push DSP Image to the Device

MTK Flashtool can be used to download a full load or update DSP binary only. There is also an open source tool “Fastboot” can be used to update DSP binary only. The command is:

- $ adb reboot bootloader  (put the device in fastboot mode)

- $ flashboot.exe scp1 tinysys-scp.bin (update scp1 from local tinysys-scp.bin)

- $ flashboot.exe scp2 tinysys-scp.bin

- Parameter Management

  - Parameter File

The parameters of different modes are gathered into a file. To get the maximum flexibility of use, the format of the parameter file is defined by the 3rd Parties themselves. The file should be placed in the following path: /vendor/mediatek/proprietary/external/aurisys/. Then the parameter files are parsed by the 3rd Party parameter parsing library to get required parameters

  - Parser Library

    - Development

Follow ARSI and implement the functions defined in arsi_api.h to develop the parser library.  Notice that both 32-bit library and 64-bit library are required. The built *.so would be placed in vendor/mediatek/proprietary/external/aurisys/phone_call

    - Configuration

The /vendor/mediatek/proprietary/external/aurisys/aurisys_device.mk should also be modified as follows to add the parser library and parameter file:

ifeq ($(MTK_AURISYS_PHONE_CALL_SUPPORT),yes) PRODUCT_PACKAGES += lib_xxx PRODUCT_COPY_FILES += vendor/mediatek/proprietary/external/aurisys/[vendor]/vendor.dat:system/etc/aurisys_param/vendor.dat endif

- Tool Development

The connection between the PC tool and the mobile device is through USB and the communication is based on adb commands. We’ve defined a series of adb commands. The software IP providers can design their own PC tools which can communicate with the mobile devices without making changes to the legacy code of the MTK platforms. The detail information can be found in the next chapter.

#### Brief Summary

1. Configurations

a) Enable Aurisys phone call supporting configurations in both Application processor and DSP.

b) Adjust heapsize of DSP

b) Estimate clock rate requirement and modify the value………………..reference to [7.1]

2. Software IP Development

a) Implement the functions in ARSI………………..reference to [3.2]

b) Build DSP binary only or build full load (include application processor) …reference to [ 7.2]

c) Update DSP binary by Fastboot or Flashtool, or update full Anroid load by Flashtool…reference to [ 7.2]

3. Parameter Management………………..reference to [3.3.3]

a) Define own parameter file format

b) Implement Parameter Parser Library

4. Tool Development………………..reference to [3.3.2]

a) Develop a PC tool. The communication between the device is based on adb commands

### Integrate Software IP in Application Processor

#### Integration of a Record SWIP (as example)

- Configurations

  - AP Configurations

As mentioned above, the file “device/mediatek/[project]/ProjectConfig.mk” is used to enable or disable a feature for a specified project of application processor. To enable Auisys framework in the application processor, please set MTK_AURISYS_FRAMEWORK_SUPPORT as “yes”. If this configuration is set as “no”, we still support MTK proprietary library in the original framework. However, it doesn’t support the XML descriptor mentioned later or support aurisys interface. It will be hard to replace the proprietary library with vendor’s library.

- Processing and Parser Library

Please refer to the parser library part of integrating software IP in DSP mentioned above. The only difference is that the processing library also exists in the application processor and you can merge processing and parser capabilities in the same library.

Besides, even the integration is complete, you may still need to do some experiments. By using the adb command “adb push xxx.lib /system/lib”, It is allowed to replace the library without re-compiling the image.

- XML Description File

  - File Path: /vendor/mediatek/proprietary/external/aurisys/aurisys_config.xml

The Aurisys XML Descriptor is a simple XML-file that specifies the library version number, attribute, parameter path, and optionally other information. It contains the library descriptor and scenario descriptor. The library descriptor is used to describe the library name, path, attributes. The scenario descriptor is used to specify which libraries would be applied in that scenario. The detailed description of XML descriptor is shown in the “XML Descriptor for Aurisys” chapter.

Besides, it is worth to be mentioned that you can also replace the XML descriptor by a adb command during development stage. The command is “adb push aurisys_config.xml /system/etc”

- Aurisys Framework Modifications

  - Assign a Scenario to the Scene Handler

As mentioned above, a scenario descriptor in XML is used to describe which libraries will be applied in that scenario. Moreover, a scene handler is created as the middleware between the audio system and the sound processing IPs. For example, the record handler is served for a record scene. We need to assign a scenario to the record scene handler that the specified scenario will be applied during recording. To specify a scenario to the record scene handler, please find the following file: “vendor/mediatek/proprietary/hardware/audio/common/V3/aud_drv/ AudioALSACaptureDataClientNormal.cpp”. Then modify the scenario in the constructor:

AudioALSACaptureDataClientNormal::AudioALSACaptureDataClientNormal(……) { char *scenario = "xxx_record_scenario"; //the scenario name }

  - Link library

    - All libraries should be linked first. It is required to let the system be aware of which libraries would be linked. First, declare a function pointer assignment interface in the following file: “external/aurisys/interface/Arsi_libary_entry_points.h”. Add an unique interface here and implement it in your library so that we can add multiple libraries at the same time.

void iir_arsi_assign_lib_fp(AurisysLibInterface *lib); void audioloud_arsi_assign_lib_fp(AurisysLibInterface *lib);

    - The second step is to link the library by calling the unique function defined. Please find the file:” /vendor/mediatek/proprietary/hardware/audio/common/aurisys/framework/AudioAurisysLibDescriptorc.c”. And then add a section to link the specified library.

static int LinkLibFunction(struct AurisysLibDescriptorc *self) { string_buf_t lib_version; status_t ret = NO_ERROR; if (strncmp(self->mName, "mtk_iir", strlen(self->mName)) == 0) { memset(&self->mAuriInterface, 0, sizeof(AurisysLibInterface)); iir_arsi_assign_lib_fp(&self->mAuriInterface); // check entry point AUD_ASSERT(self->mAuriInterface.arsi_query_working_buf_size != NULL); // get api lib_version.memory_size = 128; lib_version.string_size = 0; lib_version.p_string = (char *)AUDIO_MALLOC(lib_version.memory_size); memset(lib_version.p_string, 0, sizeof(lib_version.memory_size)); ret = self->mAuriInterface.arsi_get_lib_version(&lib_version); if (ret == NO_ERROR) { if (lib_version.string_size == 0 ¦¦ lib_version.string_size >= lib_version.memory_size) { AUD_LOG_W("lib_version bug, string_size = %u, memory_size = %u", lib_version.string_size, lib_version.memory_size); } else { AUD_LOG_D("lib_version: %s", lib_version.p_string); } } AUDIO_FREE(lib_version.p_string); } }

  - Set a Keyword for Set/Get Parameters

As mentioned, we’ve defined a series of adb commands for tuning and updating parameters. It needs to be aware of which libraries should be called to update the parameters. Thus, we need to define the keyword first (the adb commands will be described later). Please find the file “aurisys_lib_manager.c” located at the path: “vendor/mediatek/proprietary/hardware/audio/common/aurisys/framwork”. And then add the keyword to the following functions:

static status_t set_parameter(AurisysLibManager *self, const char *key_value_pair)

static char *get_parameter(AurisysLibManager *self, const char *key)

#### Brief Summary

1. Configurations

a) Enable MTK_AURISYS_FRAMEWORK_SUPPORT configurations in the project configuration file.

2. Software IP Development and Parameter Management

a) Implement the functions in ARSI………………..reference to [3.2]

b) Define own parameter file format ………………..reference to [3.3.3]

c) Implement Parameter Parser Library

d) Place the library in vendor/mediatek/proprietary/external/aurisys and modify the related Makefile

3. XML Descriptor………………..reference to [4]

a) Add a library descriptor about the specified library

b) Add a scenario descriptor to apply the library in that scenario

4. Aurisys Framework Modifications………………..reference to [2.3.2.1]

a) Assign a scenario defined in the XML descriptor to the specified scene handler

b) Declare and implement a unique API(for the Aurisys framework to invoke the library functions)

c) Define a keyword to represent a specified library so that the Aurisys framework can be aware of

which library should be called. (When there is an adb command applied to set/get parameters)

5. Tool Development………………..reference to [3.3.2]

a) Develop a PC tool. The communication between the device is based on adb commands

# Software Development Guidance

## Software Interface Overview

ARSI consists of the following components: ARSI-PROC and ARSI-PPAR. ARSI-PROC defines generic interfaces for processing sounds which is portable to MTK platforms, regardless of DSP or MCU. ARSI-PAR defines API for parsing parameters. It provides programming interface to parse and transmit parameters between PC tool, APMCU, and DSP. Tuning and applying parameters by scenes can also be achieved.

## Sound Processing Task Integration Guidance

### ARSI-PROC

It defines generic interfaces for processing sounds which is portable to MTK platforms, regardless of DSP or MCU. The interfaces include: Updating parameters and device information; Querying IP information (such as memory usage); Changing mode; Processing sounds. The interfaces are consistent among different processors. The software could be portable to reduce the design effort significantly. They are also consistent among different activities so that they can be widely used in many applications, such as playing, recording, voice call, and sound enhancing.

### ARSI-PROC Functions

| Functions | Description |
| --- | --- |
| arsi_query_working_buf_size | Query the size of the working buffer |
| arsi_create_handler | Create handler and initialize it |
| arsi_process_ul_buf | Processing microphone/uplink data |
| arsi_process_dl_buf | Processing playback/downlink data |
| arsi_destroy_handler | Deinitialize handler and destroy it (no need to free the working buffer) |
| arsi_update_device | Update task device info |
| arsi_update_param | Update speech enhancement parameters |
| arsi_set_ul_digital_gain | Set uplink digital gain |
| arsi_set_dl_digital_gain | Set downlink digital gain |
| arsi_set_ul_mute | Mute/unmute uplink |
| arsi_set_dl_mute | Mute/unmute downlink |
| arsi_set_ul_enhance | Enable/disable uplink enhancement function |
| arsi_set_dl_enhance | Enable/disable downlink enhancement function |
| arsi_set_debug_log_fp | Set debug log print callback function |

We’ll introduce ARSI-PROC functions here and provide examples. Notice that we won’t provide detailed descriptions of all ARSI-PROC functions. Please refer to the interface document generated by doxygen.

#### arsi_process_ul_buf

Processing microphone/ voice uplink data

- Prototype:

lib_status_t arsi_process_ul_buf(audio_buf_t  *p_ul_buf_in,  audio_buf_t    *p_ul_buf_out,  audio_buf_t    *p_aec_buf_in, const uint32_t  delay_ms, void *p_handler, void *arg);

- Parameters:

| Parameters | Description |
| --- | --- |
| *p_ul_buf_in | microphone/uplink data to be processed buffer arrangement: chunks of audio destined for different channels => mic1(1 frame sz) + ... + micN (1 frame sz) |
| *p_ul_buf_out | the processed microphone/uplink data |
| *p_aec_buf_in | the AEC reference data |
| delay_ms | delay time(ms) for AEC |
| *p_handler | handler of speech enhancement |
| *arg | reserved field (can be used for extension) |

- Return value:

lib_status_t, the status of library

#### arsi_process_dl_buf

Processing playback/voice downlink data

- Prototype:

lib_status_t arsi_process_dl_buf(audio_buf_t *p_dl_buf_in, audio_buf_t *p_dl_buf_out,  void *p_handler, void *arg);

- Parameters:

| Parameters | Description |
| --- | --- |
| *p_dl_buf_in | the playback/downlink data to be Processed |
| *p_dl_buf_out | the processed playback/downlink data |
| *p_handler | handler of speech enhancement |
| *arg | reserved field (can be used for extension) |

- Return value:

lib_status_t, the status of library

### Example

Init() { …… arsi_create_handler(&arsi_task_config, NULL/*&param_buf*/, &working_buf, &arsi_handler); arsi_set_debug_log_fp(myprint, arsi_handler); arsi_set_ul_digital_gain(0, 0, arsi_handler); arsi_set_dl_digital_gain(0, 0, arsi_handler); arsi_set_ul_mute(0, arsi_handler); // no bool.... 0: false, 1: true arsi_set_dl_mute(0, arsi_handler); // no bool.... 0: false, 1: true arsi_set_ul_enhance(1, arsi_handler); // no bool.... 0: false, 1: true arsi_set_dl_enhance(1, arsi_handler); // no bool.... 0: false, 1: true …… } Process() { …… const uint32_t aec_delay_ms = 40; if (modem_data_handshake == MODEM_UL_DATA) { arsi_process_ul_buf( &ul_buf_in, &ul_buf_out, &aec_buf_in, aec_delay_ms, arsi_handler, (void *)&extra_call_arg); } else if (modem_data_handshake == MODEM_DL_DATA) { arsi_process_dl_buf( &dl_buf_in, &dl_buf_out, arsi_handler, (void *)&extra_call_arg); } …… }

## Parameter Tuning Guidance

Figure 3-1. Parameter Path

### ARSI-PAR Usage

For a software IP, the parameters may be variant in different scenes or situations. To ease of reference, The parameters are gathered into a file. To get the maximum flexibility of use, the format of the parameter file is defined by the 3rd Parties themselves. The ARSI-PAR is designed for parsing parameters for specified scenes. It provides  the interface to transfer parameter file to the 3rd party library.

- Tool Development / Connection

The connection between the PC tool and the mobile device is through USB and the communication is based on adb commands. We define a series of adb commands which include transmitting the whole parameter file to the device, receiving whole parameter file from the device, transmitting specific data to the device, and receiving specific data from the device. The software IP providers can design their own PC tools which can communicate with the mobile devices without making changes to the legacy code of the MTK platforms.

- Parsing

To get the maximum flexibility of use, the format of the parameter file is also defined by the 3rd Parties themselves. We only provide the interface to transfer parameter file to the 3rd Party Library. Thus, the 3rd Parties need to implement the parameter parsing library.

- Tuning/Extraction

During parameters tuning, the PC tool updates the parameter file to the mobile device and then the parameter files are parsed by the 3rd Party parameter parsing library to get required parameters. The parameters will then be transferred to the 3rd Party processing library for processing sounds. Besides, when the mode changes (for example, the output device changes from headphone to speaker), we need to extract the parameters of the specified mode and the parsing library will be called.

### Communication based on Android Debug Bridge(adb)

Table 3-1. Adb command table

| Command | Comments |
| --- | --- |
| adb shell "AudioSetParam AURISYS_SET_PARAM,$target,$scene,$lib,$cmd $mode=SET” | Used to update parameter file and apply parameters to the library |
| adb shell “AudioSetParam AURISYS_GET_PARAM,$target,$scene,$lib,$cmd” | Used to query current parameter setting. Query the value of a specified address and query the whole parameter file are allowed. |

Table 3-2.Command argument table

| Argument | Example | Comments |
| --- | --- | --- |
| $target | DSP, HAL | The processing API maybe either in AP or in DSP |
| $scene | PHONE_CALL, PLAYBACK, RECORD | Specify a scene, the information will be used as an argument of the parser function |
| $lib | library name | The library should be specified since there maybe several libraries added to the Aurisys |
| $mode | Enhancement mode | Enhancement mode. IP provider can provide different enhancement for user to select what they want. The mode should be an integer. Only used in APPLY_PARAM command |
| $cmd (for URISYS_SET_PARAM) | PARAM_FILE,param_path | set parameter file path |
|  | APPLY_PARAM | apply new param during playback/phone call/... |
|  | ADDR_VALUE,addr,value | set “value” at the “addr” in library |
|  | KEY_VALUE,key,value | set "value" of the "key" in library, “key” can be defined by the library itself |
| $cmd (for URISYS_GET_PARAM) | PARAM_FILE | return param_path |
|  | ADDR_VALUE,addr | return the value at the address “addr” in library |
|  | KEY_VALUE,key | return the value at the “key” in library |

The examples are listed as follows:

- adb shell "AudioSetParam AURISYS_SET_PARAM,DSP,RECORD,MyLib,PARAM_FILE,/data/d.dat=SET"

set parameter file path: /data/d.dat and establish the connection between MyLib and the parameter file

- adb shell "AudioSetParam AURISYS_SET_PARAM,DSP,PHONE_CALL,MyLib,APPLY_PARAM,2=SET"

apply new parameters for record scene, the enhancement mode is 2

- adb shell "AudioSetParam AURISYS_SET_PARAM,DSP,RECORD,MyLib,ADDR_VALUE,0x1234,0x5678=SET" set value 0x5678 at the address 0x1234 in MyLib

- adb shell "AudioSetParam AURISYS_SET_PARAM,DSP,RECORD,MyLib,KEY_VALUE,MyKey,MyVauel=SET"

set value "MyValue" to the key "MyKey" in MyLib

- adb shell "AudioSetParam AURISYS_GET_PARAM,DSP,PHONE_CALL,MyLib,PARAM_FILE"

return param_path of MyLib

- adb shell "AudioSetParam AURISYS_GET_PARAM,DSP,PHONE_CALL,MyLi,ADDR_VALUE,0x1234"

return the value at the address 0x1234 in Mylib

- adb shell "AudioSetParam AURISYS_GET_PARAM,DSP,PHONE_CALL,MyLib,KEY_VALUE,MyKey"

return the value of the key "MyKey" in Mylib

### ARSI-PAR Functions

| Functions | Description |
| --- | --- |
| arsi_query_param_buf_size | Query the buffer size to keep speech enhancement parameters(single mode), Implemented in HAL only |
| #### arsi_parsing_param_file | Parsing param file to get parameters into p_param_buf(single mode), Implemented in HAL only |
| arsi_set_addr_value | Set value at a specified address |
| arsi_get_addr_value | Get value from the specified address |
| arsi_set_key_value_pair | set key_value string to library |
| arsi_get_key_value_pair | get key_value string from library |

We’ll introduce ARSI-PAR functions here and provide examples. Notice that we won’t provide detailed descriptions of all ARSI-PAR functions. Please refer to the interface document generated by doxygen.

#### arsi_query_param_buf_size

Query the buffer size to keep speech enhancement parameters(single mode), Implemented in HAL only

- Prototype:

lib_status_t arsi_query_param_buf_size( const arsi_task_config_t *p_arsi_task_config, const string_buf_t       *platform_name, const string_buf_t *param_file_path, const int enhancement_mode, uint32_t *p_param_buf_size);

- Parameters:

| Parameters | Description |
| --- | --- |
| *p_arsi_task_config | the task configure |
| *platform_name | the platform name by "adb shell getprop ro.product.model" |
| *param_file_path | the speech enhancement param file (fullset) |
| enhancement_mode | the speech enhancement mode by apk |
| *p_param_buf_size | How much memory size required to keep the enhancement parameters for the specific device/mode |

- Return value:

lib_status_t, the status of library

#### arsi_parsing_param_file

Parsing param file to get parameters into p_param_buf(single mode), Implemented in HAL only

- Prototype:

lib_status_t arsi_parsing_param_file( const arsi_task_config_t *p_arsi_task_config, const string_buf_t *platform_name,  const string_buf_t *param_file_path,  const int enhancement_mode,  data_buf_t *p_param_buf);

- Parameters:

| Parameters | Description |
| --- | --- |
| *p_arsi_task_config | the task configure |
| *platform_name | the platform name by "adb shell getprop ro.product.model" |
| *param_file_path | the speech enhancement param file (fullset) |
| enhancement_mode | the speech enhancement mode by apk |
| * p_param_buf | the buffer pointer of the parameters for the specific device/mode |

- Return value:

lib_status_t, the status of library

### Example

void *handle = dlopen(lname, RTLD_NOW); …… arsi_query_param_buf_size_t fvsoft_arsi_query_param_buf_size = (arsi_query_param_buf_size_t)dlsym(handle, "arsi_query_param_buf_size"); …… arsi_parsing_param_file_t fvsoft_arsi_parsing_param_file = (arsi_parsing_param_file_t)dlsym(handle, "arsi_parsing_param_file"); …… fvsoft_arsi_query_param_buf_size(&mArsiTaskConfig, &platform_name, &file_path, 0, &param_buf_size); memset(param_buf.p_buffer, 0, param_buf.memory_size); fvsoft_arsi_parsing_param_file(&mArsiTaskConfig, &platform_name, &file_path, 0, &param_buf); …… /* set speech param to SCP */ pIPI->sendIpiMsg(TASK_SCENE_PHONE_CALL, AUDIO_IPI_DMA, AUDIO_IPI_MSG_NEED_ACK, IPI_MSG_A2D_SPH_PARAM, param_buf.data_size, 0, (char *)param_buf.p_buffer); dlclose(handle);

# XML Descriptor for Aurisys

### Example

The Aurisys XML Descriptor is a simple XML-file that specifies the library version number, attribute, parameter path, and optionally other information. We provide an example, in the following demonstrates how to describe a library and a scenario. The attributes of the MTK IIR filter library and the MTK record library are packaged in the library descriptor by <aurisyslibdescriptor> tag. Then we can use <aurisysscenario> tag to describe which library (can be multiple libraries) will be applied in this scenario.

<aurisysconfig> <versioncontrol value="1.01"> </versioncontrol> <!-- mtk iir lib descriptor --> <aurisyslibdescriptor name="library_iir"> <aurisyslibAttribute name="AurisysLibPath" value="/system/lib/lib_iir.so"/> <aurisyslibAttribute name="AurisysParamPath" value=""/system/etc/aurisys_param/iir.dat > <aurisyslibAttribute name="AurisysLibVersion" value="001"/> <aurisyslibAttribute name="AurisysLibLayer" value="AP"/> <aurisyslibAttribute name="AurisysLibScenario" value="TASK_SCENE_PHONE_CALL¦TASK_SCENE_RECORD¦TASK_SCENE_VOIP"/> <aurisyslibAttribute name="AurisysLibKeyFrameMs" value="5¦20"/> <aurisyslibAttribute name="AurisysLibKeyparam" value="iir"/> <aurisyslibAttribute name="AurisysLibBufferbase" value="FRAMEBASE"/> <aurisyslibAttribute name="AurisysLibCapability" value="UPLINK"/> <!—stream descriptor for mtk iir lib --> <aurisysStreamdescriptor> <aurisysStreamAttribute name="AurisysLibULDevice" value="AUDIO_DEVICE_IN_BUILTIN_MIC¦AUDIO_DEVICE_IN_WIRED_HEADSET¦AUDIO_DEVICE_IN_BLUETOOTH_SCO_HEADSET"/> <aurisysStreamAttribute name="AurisysLibULDeviceExt" value="rec_mic"/> <aurisysStreamAttribute name="AurisysLibULFormatInput" value="AUDIO_FORMAT_PCM_16_BIT¦AUDIO_FORMAT_PCM_8_24_BIT¦AUDIO_FORMAT_PCM_32_BIT"/> <aurisysStreamAttribute name="AurisysLibULSamplerateInput" value="16000¦32000¦48000¦96000¦192000"/> <aurisysStreamAttribute name="AurisysLibULChannelInput" value="1¦2"/> <aurisysStreamAttribute name="AurisysLibULFormatOutput" value="AUDIO_FORMAT_PCM_16_BIT¦AUDIO_FORMAT_PCM_8_24_BIT¦AUDIO_FORMAT_PCM_32_BIT"/> <aurisysStreamAttribute name="AurisysLibULSamplerateOutput" value="16000¦32000¦48000¦96000¦192000"/> <aurisysStreamAttribute name="AurisysLibULChannelOutput" value="1¦2"/> </aurisysStreamdescriptor> </aurisyslibdescriptor> <!-- mtk record lib descriptor --> <aurisyslibdescriptor name="library_record"> <aurisyslibAttribute name="AurisysLibPath" value="/system/lib/libspeech_enh_lib.so"/> <aurisyslibAttribute name="AurisysParamPath" value="/system/etc/aurisys_param/mtk_record.dat "/> <aurisyslibAttribute name="AurisysLibVersion" value="001"/> <aurisyslibAttribute name="AurisysLibLayer" value="AP"/> <aurisyslibAttribute name="AurisysLibScenario" value="TASK_SCENE_RECORD"/> <aurisyslibAttribute name="AurisysLibKeyFrameMs" value="1¦3¦5¦10¦20"/> <aurisyslibAttribute name="AurisysLibKeyparam" value="record"/> <aurisyslibAttribute name="AurisysLibBufferbase" value="FRAMEBASE"/> <aurisyslibAttribute name="AurisysLibCapability" value="UPLINK"/> <!—stream descriptor for mtk record lib --> <aurisysStreamdescriptor> <aurisysStreamAttribute name="AurisysLibULDevice" value="AUDIO_DEVICE_IN_BUILTIN_MIC¦AUDIO_DEVICE_IN_WIRED_HEADSET¦AUDIO_DEVICE_IN_BLUETOOTH_SCO_HEADSET"/> <aurisysStreamAttribute name="AurisysLibULDeviceExt" value="rec_mic"/> <aurisysStreamAttribute name="AurisysLibULFormatInput" value="AUDIO_FORMAT_PCM_16_BIT"/> <aurisysStreamAttribute name="AurisysLibULSamplerateInput" value="48000"/> <aurisysStreamAttribute name="AurisysLibULChannelInput" value="1¦2"/> <aurisysStreamAttribute name="AurisysLibULFormatOutput" value="AUDIO_FORMAT_PCM_16_BIT"/> <aurisysStreamAttribute name="AurisysLibULSamplerateOutput" value="48000"/> <aurisysStreamAttribute name="AurisysLibULChannelOutput" value="1¦2"/> </aurisysStreamdescriptor> </aurisyslibdescriptor> <!—scenario descriptor, record scenario, use mtk record library --> <aurisysscenario name="inhouse_record"> <aurisysscenariolib name="library_record"/> <aurisysscenariolib name="library_iir"/> </aurisysscenario> </aurisysconfig>

### Elements

#### Version Control

The <versioncontrol> tag is used to describes the Aurisys XML format version.

<versioncontrol value="1.01">

#### Library Descriptor

- Library Name

The library descriptor is used to describe the information of a library, such as library version, library path, and processing approaches. You can create a library descriptor by a <aurisyslibdescriptor> tag with an appropriate name which represents the library.

<aurisyslibdescriptor name="library_iir">

- Library Path

Specify the library path.

<aurisyslibAttribute name="AurisysLibPath" value="/system/lib/lib_iir.so"/>

- Parameter Path

Specify the library parameter file path. The usage of library parameter has been demonstrated in the chatper “Software Development Guidance”

<aurisyslibAttribute name="AurisysParamPath" value=""/system/etc/aurisys_param/iir.dat >

- Library Version Control

Version of the library. There is no definition of the value range, however, it should be an integer.

<aurisyslibAttribute name="AurisysLibVersion" value="001"/>

- Library Layer

As mentioned above, the library can be processed in both DSP and AP. This tag is used to specify it is processed in AP or in DSP.

  - Range of Values: AP/DSP

<aurisyslibAttribute name="AurisysLibLayer" value="AP"/>

- Library Applied Scenario

Specify the library can be applied in which scenario.

  - Range of values: TASK_SCENE_PHONE_CALL,TASK_SCENE_VOW , TASK_SCENE_PLAYBACK_MP3 , TASK_SCENE_PLAYBACK,TASK_SCENE_RECORD,TASK_SCENE_VOIP, TASK_SCENE_SPEAKER_PROTECTION

<aurisyslibAttribute name="AurisysLibScenario" value="TASK_SCENE_PHONE_CALL¦TASK_SCENE_RECORD¦TASK_SCENE_VOIP"/>

- Frame Period

The supported frame period of the library. For example, 5|20 means the library support both 5ms and 20ms.

  - Range of values: an integer value

<aurisyslibAttribute name="AurisysLibKeyFrameMs" value="5¦20"/>

- Key Parameter

When we want to modify the parameters (ex. by a tuning tool), the audio system needs a specified key to recognize which library should be applied. The key can be any string value that is unique.

<aurisyslibAttribute name="AurisysLibKeyparam" value="iir"/>

- Processing Approach

The library can be interleave and framebase, frame base need to assign AurisysLibKeyFrameMs

  - Range of values: SAMPLEBASE, FRAMEBASE

<aurisyslibAttribute name="AurisysLibBufferbase" value="FRAMEBASE"/>

- Capability

Specify which types of data buffer should be provided to the library. For example, UPLINK|DLREFERENCE mean the library requires the uplink data and downlink as reference buffer.

  - Range of values: DOWNLINK, UPLINK , DLREFERNENCE , ECHOREF

<aurisyslibAttribute name="AurisysLibCapability" value="UPLINK"/>

#### Stream Descriptor

The stream descriptor is used to specify the supported stream format (bit resolution, sampling rate, etc.,.). There are downlink, uplink, and reference stream types. You don’t need to fill downlink stream information if the library doesn’t support a downlink stream. The tags <aurisysStreamdescriptor>, </aurisysStreamdescriptor> is used to define the start and end of the stream descriptor section.

- Primary Sections

  - Device

To specify the supported output devices for a downlink stream and the supported input devices for a uplink stream.

    - Range of values for DL: AUDIO_DEVICE_OUT_EARPIECE, AUDIO_DEVICE_OUT_SPEAKER, AUDIO_DEVICE_OUT_WIRED_HEADSET, AUDIO_DEVICE_OUT_WIRED_HEADPHONE, AUDIO_DEVICE_OUT_BLUETOOTH_SCO

    - Range of values for UL: AUDIO_DEVICE_IN_BLUETOOTH_SCO_HEADSET, AUDIO_DEVICE_IN_WIRED_HEADSET, AUDIO_DEVICE_IN_BUILTIN_MIC

<aurisysStreamAttribute name="AurisysLibULDevice" value="AUDIO_DEVICE_IN_BUILTIN_MIC¦AUDIO_DEVICE_IN_WIRED_HEADSET¦AUDIO_DEVICE_IN_BLUETOOTH_SCO_HEADSET"/> <aurisysStreamAttribute name="AurisysLibDLDevice" value="DeviceOutReceiver¦AUDIO_DEVICE_OUT_SPEAKER¦DeviceOutHeadphone¦AUDIO_DEVICE_OUT_BLUETOOTH_SCO"/>

  - Physical Device

The physical intput/output devices. However, the property is not used currently.

    - Range of values:

<aurisysStreamAttribute name="AurisysLibULDeviceExt" value="rec_mic"/> <aurisysStreamAttribute name="AurisysLibDLDeviceExt" value="voip_spk"/>

  - Input Data Format

Specify the supported bit resolution of the input data to the library. The range of values is the same for DL and UL stream.

    - Range of values: AUDIO_FORMAT_PCM_16_BIT, AUDIO_FORMAT_PCM_24_BIT, AUDIO_FORMAT_PCM_32_BIT

<aurisysStreamAttribute name="AurisysLibULFormatInput" value="AUDIO_FORMAT_PCM_16_BIT¦AUDIO_FORMAT_PCM_8_24_BIT¦AUDIO_FORMAT_PCM_32_BIT"/> <aurisysStreamAttribute name="AurisysLibDLFormatInput" value="AUDIO_FORMAT_PCM_16_BIT"/>

  - Input Sample Rate

Specify the supported sample rate of the input data to the library. The range of values is the same for DL and UL stream.

    - Range of values: 8000, 16000, 32000, 44100, 48000, 88200, 96000, 176400, 192000

<aurisysStreamAttribute name="AurisysLibULSamplerateInput" value="16000¦32000¦48000¦96000¦192000"/> <aurisysStreamAttribute name="AurisysLibDLSamplerateInput" value="48000"/>

  - Input Channel Number

Specify the supported channel number of the input data to the library. The range of values is the same for DL and UL stream.

    - Range of values: 1, 2

<aurisysStreamAttribute name="AurisysLibULChannelInput" value="1¦2"/> <aurisysStreamAttribute name="AurisysLibDLChannelInput" value="1"/>

  - Output Data Format

Specify the supported bit resolution of the output data to the library. The range of values is the same for DL and UL stream.

    - Range of values: AUDIO_FORMAT_PCM_16_BIT, AUDIO_FORMAT_PCM_24_BIT, AUDIO_FORMAT_PCM_32_BIT

<aurisysStreamAttribute name="AurisysLibULFormatOutput" value="AUDIO_FORMAT_PCM_16_BIT¦AUDIO_FORMAT_PCM_8_24_BIT¦AUDIO_FORMAT_PCM_32_BIT"/> <aurisysStreamAttribute name="AurisysLibDLFormatOutput" value="AUDIO_FORMAT_PCM_16_BIT"/>

  - Output Sample Rate

Specify the supported sample rate of the output data to the library. The range of values is the same for DL and UL stream.

    - Range of values: 8000, 16000, 32000, 44100, 48000, 88200, 96000, 176400, 192000

<aurisysStreamAttribute name="AurisysLibULSamplerateOutput" value="16000¦32000¦48000¦96000¦192000"/> <aurisysStreamAttribute name="AurisysLibDLSamplerateOutput" value="48000"/>

  - Output Channel Number

Specify the supported channel number of the input data to the library. The range of values is the same for DL and UL stream.

    - Range of values: AUDIO_FORMAT_PCM_16_BIT, AUDIO_FORMAT_PCM_24_BIT, AUDIO_FORMAT_PCM_32_BIT

<aurisysStreamAttribute name="AurisysLibULChannelOutput" value="1¦2"/> <aurisysStreamAttribute name="AurisysLibDLChannelOutput" value="1"/>

- Optional Sections

  - Reference Stream Data Format

Specify the supported bit resolution of the reference stream to the library. The range of values is the same for DL and UL reference stream.

    - Range of values: AUDIO_FORMAT_PCM_16_BIT, AUDIO_FORMAT_PCM_24_BIT, AUDIO_FORMAT_PCM_32_BIT

<aurisysStreamAttribute name="AurisysLibDLFormatRef" value="1¦2"/> <aurisysStreamAttribute name="AurisysLibULFormatRef" value="1¦2"/>

  - Reference Stream Data Sample Rate

Specify the supported sample rate of the reference stream to the library. The range of values is the same for DL and UL reference stream.

    - Range of values: 8000, 16000, 32000, 44100, 48000, 88200, 96000, 176400, 192000

<aurisysStreamAttribute name="AurisysLibDLSamplerateRef" value=""/> <aurisysStreamAttribute name="AurisysLibULSamplerateRef" value=""/>

  - Reference Stream Data Channel Number

Specify the supported channel number of the reference stream to the library. The range of values is the same for DL and UL reference stream.

    - Range of values:1, 2

<aurisysStreamAttribute name="AurisysLibDLChannelRef" value=""/> <aurisysStreamAttribute name="AurisysLibULChannelRef" value=""/>

#### Scenario Descriptor

The scenario descriptor is used to specify which library would be applied in the scenario. The tags < aurisysscenario>, </aurisysscenario> is used to define the start and end of the scenario descriptor section. The following example shows that an inhouse_record scenario is created and there are two libraries will be applied in the scenario. Then we should assign the inhouse_record scenario to be applied in the record scene. There is a detailed description later.

<aurisysscenario name=" inhouse_record"> <aurisysscenariolib name="library_record"/> <aurisysscenariolib name="library_iir"/> </aurisysscenario>

### Deployment

Figure 4-1. Relationship between Scene Handler, Scenario Descriptor, and Library Descriptor

Library Descriptor is used to describe the information of a library. Every applied library should be described by a library descriptor.

Scenario Descriptor is used to define which libraries would be applied in the specified scenario. Multiple libraries can be applied in one scenario. Besides, as the diagram above, a library can be applied in different scenarios.

Scene Handler, as mentioned above, is created as the middleware between the audio system and the sound processing IPs. The record handler is served for a record scene and we can assign a scenario to the record handler. If we assign the scenario A to the record handler, the library A and library B will be applied in the record scene. We can also assign scenario B or scenario C to the recording scene. However, the assignment is achieved in the program of scene handler, not in the XML descriptor.

# Debugging

## Logging

Programmers are used to debug by logging the execution of the program. However, it is not convenient for a binary release IP. During developing an IP, the programmer writes logging codes to check the correctness of the program. However, the logging code may be removed when it’s time to release. Moreover, the programmer may want to refine the program due to issues or improvement works. He needs to insert the logging code and recompile the code. This brings lots of work for the developer, especially when the program is binary released.

Thus, we provide an API for the developer to check the correctness  of their program by logging. The API is consistent and can be used in different systems. The provided API would call the function pointer passed in the initialization. The called function is responsible for logging the required statements by calling the system-specific logging API. It can also be disabled easily by bypassing the requirements. It’s helpful for the developer which doesn’t need to remove and insert logging code anymore.

### Inside Software IP

The following blocks are the statement of the information about the provided logging API:

- API

/** ****************************************************************************** * @brief set debug log print callback function * * @param debug_log log print function pointer * * @param p_handler the handler of speech enhancement * * @return lib_status_t ****************************************************************************** */ lib_status_t arsi_set_debug_log_fp(const debug_log_fp_t debug_log, void *p_handler);

- Prototype

typedef void (*debug_log_fp_t)(const char *message, ...);

- Data Structure

typedef struct arsi_task_config_t { … debug_log_fp_t debug_log; … } arsi_task_config_t;

# Applications

## Voice Call

### Data Path

- Hand-Held Mode

(6)

(5)

(1)

(4)

(3)

(2)

Figure 6-1. Hand-Held Mode Voice Call Data Path

- Downlink data. The data from the far-end is received in Modem IC and decoded. After decoding the codec, the Modem IC notifies the Open DSP to move the voice data to the internal memory of Open DSP. Then the vendor’s DL enhancement solution will be applied and the processed data would be moved back to Modem IC.

- The Modem IC write DL data to Audio HW.

- The voice uplink data are also received from the Modem IC.

- The Open DSP moves the data to its internal memory. After applying the vendor’s UL enhancement solution, the data will be transferred to Modem IC. The Modem IC will then encode the data and send to far end.

- The background sound is transferred to Modem IC and mixed with voice data before output to the audio hardware

- The voice data during a voice call can be saved. The DL/UL data can be saved separately.

- Hand-Free Mode

Programmers are used to debug by logging the execution of the program. However, it is not convenient for

(3)

(2)

(1)

Figure 6-2. Hand-Free Mode Voice Call Data Path

- If there is no smartPA, the DL data is written to the Audio HW  and output to speaker Amp directly. If there is a smartPA, the output data will be routed to the application processor first

- The smartPA algorithm will be applied. Notice that we only support smartPA algorithm by default in MT6797.

- Finally, the data will be written to the Audio HW and then output to the speaker Amp.

### Basic Information

- We support NB/WB. No matter NB or WB, the data will be up-sampled to 16k sampling rate for the vendor’s IP. We’ll notify the vendor’s IP whether it’s WB or not.

- 20ms per frame and DL/UL separated. Modem side interrupts open DSP every  20ms to notify open DSP to receive voice UL data. For voice DL data, the interrupt period is also 20ms, and there is a shift between UL data.

- To avoid increasing round trip delay, UL enhancement should be processed no more than 10ms and DL enhancement should be processed no more than 3ms. If the library can’t meet the requirement, we need to delay 1 frame to process the data. There would be 20ms increment in the round trip delay (because of 20ms per frame).

|  | Available MCPS | Latency |
| --- | --- | --- |
| DL Enhancement | 53 | No delay |
| UL Enhancement | 177 | No delay |
| DL+UL Enhancement | 354 | 20ms delay increment |

- The group delay (processing buffer delay) of the library is suggested to not more than 40ms.

- Dual mic supported in MT6797

### Limitations

The timing is critical for voice call, so the voice call enhancements should be run at 354 MHz. It will keep the voltage at 1.0V in MT6797. Compared with MTK proprietary voice call enhancement solution which is executed in the Modem IC, the power consumption of executing voice call enhancement executed in the Open DSP will be higher. However, the proportion of power increase would be small in the view of total power consumption of whole chip and daily of use.

### Configurations

AP -- MTK_AURISYS_PHONE_CALL_SUPPORT

DSP -- CFG_ MTK_AURISYS_PHONE_CALL_SUPPORT

Notice that there is a compile dependency check. The configurations of AP and DSP should be the same or there will be a build error occurs.

### Software Frameworks

(3)

(6)

(5)

(4)

(2)

(1)

Figure 6-3. Software Frameworks of Voice Call

- UL ………………………………………..……………………………………………………….…(1)

  - The diagram shows the path how UL data received from the hardware input and sends to the Modem IC

- DL………………………………………………………………………………………………..……(2)

  - The path from the Modem IC to the audio hardware output

- Data between Modem IC and CortexM4 (Open DSP) ……………………..(3)

  - The voice data exchange between the Modem IC and CM4 is by the BUS. The DMA of CM4 can help to reduce the period of data exchange

- Source code structure

  - Audio -- /source/drivers/CM4_A/mt6797/audio

  - Aurisys Interface -- /source/middleware/lib/aurisys

  - IPI -- /source/drivers/CM4_A/mt6797/ipi/

- Task Voice ………………………………………..………………………………………………(4)

  - The initial function is placed in audio.c to initialize the IPI message handler and create the Voice Task

  - The Voice Task main functions are placed in audio_task_phone_call.c

- IPI ………………………………………..………………………………………………………..…(5)

  - Inter Processor Interrupt. The communication between the application processor and the DSP is through the IPI message. Each task has its unique IPI ID and should have implemented an IPI handler.

  - The IPI ID is defined in scp_ipi.h

  - The IPI message handler and related information can be found in audio_messenger_ipi.c, audio_speech_msg_id.h

  - Notice that the IPI message handler is in ISR level. Thus, we can’t execute enhancement directly in the IPI message handler. It is just used to signal the task Voice.

- (6) Process APIs………………………………………..……………………………………(6)

  - The software interface defined in arsi_api.h. The process APIs implemented by the vendor will be executed in the Task Voice.

## Playback

### Data Path

(3)

(4)

(1)

(2)

(5)

Figure 6-4. Offload Audio Playback

- Offload stream. The audio bitstream will be decoded to DSP and the decoded PCM data will be output to the audio hardware directly.

- The normal playback stream. The audio is decoded in the application processor.

- Fast track, audio playback with lower latency.

- The decoder implemented in DSP. Currently, we only support MP3 decoder in MT6797.

- The multiple streams can be mixed in the hardware directly. However, the sampling rate should be the same, so the sampling rate converter would be executed first if needed.

### Configurations

AP -- MTK_AUDIO_TUNNELING_SUPPORT

DSP -- CFG_MTK_AUDIO_TUNNELING_SUPPORT

audio_policy.conf --

compress_offload { sampling_rates 8000¦11025¦16000¦22050¦32000¦44100¦48000 channel_masks AUDIO_CHANNEL_OUT_MONO¦AUDIO_CHANNEL_OUT_STEREO formats AUDIO_FORMAT_MP3 devices AUDIO_DEVICE_OUT_WIRED_HEADSET¦AUDIO_DEVICE_OUT_WIRED_HEADPHONE¦AUDIO_DEVICE_OUT_ANLG_DOCK_HEADSET¦AUDIO_DEVICE_OUT_DGTL_DOCK_HEADSET flags AUDIO_OUTPUT_FLAG_COMPRESS_OFFLOAD¦AUDIO_OUTPUT_FLAG_NON_BLOCKING }

### Behavior

In MT6797, We didn’t implement offloadable playback effects on DSP by default. If the playback effect is enabled, the playback will be switched to a normal stream playback. Users can implement the playback effect in DSP by themselves currently. The default package with playback effect supported in DSP will be released later.

### Functional Blocks

(8)

(7)

(6)

(4)

(5)

(3)

(2)

(1)

Figure 6-5. Functional Blocks of Offload Playback

- The communications between the application processor and DSP is by the IPI driver.

- The parsed MP3 bistream is reserved in DRAM. The application processor can enter sleep mode if there is sufficient MP3 bistrem stored in DRAM.

- DSP will move the MP3 bitstream to its internal memory by DMA

- The mp3 bistream will be decoded by the decoder

- The decoder is executed in the Task MP3

- The output PCM data will be transferred to the audio hardware by DMA

- We need to monitor the data amount of mp3 bistream buffer

- Once the amount of mp3 bitstream is not enough for decoding, we’ll interrupt the application processor by IPI message to fill the bitstream buffer

## Sound Trigger

To be released later

## SmartPA

To be released later

## Record & VOIP

Currently, we only support recording and VOIP enhancement in the Application Processor. This is because the power savings in DSP won’t be  significant unless the encoder and storage driver are also implemented in DSP. Besides, the computing and memory resource of the DSP are insufficient  for these applications.

### Data Path

(4)

(5)

(1)

(2)

(3)

Figure 6-6. Record & VOIP

- The record(uplink) stream which contains sound data from microphones.

- We also support direct input without any processing.

- Before processing the uplink data with the recording or VOIP UL library. We provide an IIR filter ahead to do DC removal and reduce possible hardware defects. It can be bypassed by XML config if you don’t need it.

- The downlink stream which is processed with a playback or VOIP DL library.

- The DL output data will be looped back as the downlink reference stream and provided to the record or VOIP UL library.

### Configurations

MTK_AURISYS_FRAMEWORK_SUPPORT

This project configuration should be set as “yes” if you want to support recording and VOIP effect in Aurisys framework. If this configuration is set as “no”, we still support MTK inside record and VOIP effect. However, we don’t support the XML  descriptor mentioned above. If you want to replace the MTK library with vendor’s library, it’s suggested to set this configuration as “yes”. We can also support MTK library in Aurisys framework in the future and set this option as true by default.

### Application Processor Only

Due to the lack of DSP resources (computing power and internal memory size). The recording and VOIP libraries are only allowed to be processed in the application processor.

### Class Diagram

The class diagram of Aurisys record and VOIP framework is depicted as follows. It provides a reference to the actual code.

Figure 6-7. Aurisys Class Diagram for Record & VOIP

### Device Information

It may be confused with the relationship between the input/output device and arguments sent to the library. We provide a comparison table in the following as a reference to implement library.

#### VOIP

task_scene = TASK_SCENE_VOIP

| Device | Device Detail | Arguments |
| --- | --- | --- |
| Handset | w/ dual-mic NR | - input_device_info = AUDIO_DEVICE_IN_BUILTIN_MIC - output_device_info = AUDIO_DEVICE_OUT_EARPIECE - VIR_VOIP_NORMAL_DMNR_SUPPORT enabled |
|  | w/o dual-mic NR | - input_device_info = AUDIO_DEVICE_IN_BUILTIN_MIC - output_device_info = AUDIO_DEVICE_OUT_EARPIECE - VIR_VOIP_NORMAL_DMNR_SUPPORT disabled |
| Headset | 3 pole headset | - input_device_info.devices = AUDIO_DEVICE_IN_BUILTIN_MIC - output_device_info.devices = AUDIO_DEVICE_OUT_WIRED_HEADPHONE |
|  | 4 pole headset | - input_device_info.devices = AUDIO_DEVICE_IN_WIRED_HEADSET - output_device_info.devices = AUDIO_DEVICE_OUT_WIRED_HEADSET - input_device_info.num_channels = 1 |
|  | 5 pole headset | - input_device_info.devices = AUDIO_DEVICE_IN_WIRED_HEADSET - output_device_info.devices = AUDIO_DEVICE_OUT_WIRED_HEADSET - input_device_info.num_channels = 2 - 5_POLE_HS_SUPPORT enabled - MTK_HEADSET_ACTIVE_NOISE_CANCELLATION_SUPPORT disabled |
|  | 5 pole + ANC | - input_device_info.devices = AUDIO_DEVICE_IN_WIRED_HEADSET - output_device_info.devices = AUDIO_DEVICE_OUT_WIRED_HEADSET - input_device_info.num_channels = 2 - MTK_HEADSET_ACTIVE_NOISE_CANCELLATION_SUPPORT enabled |
| Hand-Free | w/ NR | - output_device_info = AUDIO_DEVICE_OUT_SPEAKER - VIR_VOIP_HANDSFREE_DMNR_SUPPORT enabled |
|  | w/o NR | - output_device_info = AUDIO_DEVICE_OUT_SPEAKER - VIR_VOIP_HANDSFREE_DMNR_SUPPORT disabled |
| BT earphone | BT earphone | - input_device_info = AUDIO_DEVICE_IN_BLUETOOTH_SCO_HEADSET |

#### Record

task_scene = TASK_SCENE_RECORD

The information of input device used in recording is the same with what used in VOIP. However, the library may need to be aware of which application is processed and apply different effect according to the application. The following table lists the arguments provided to the library in each application.

| Application | Input Source | Notes |
| --- | --- | --- |
| Sound Recording | input_source = AUDIO_SOURCE_MIC | Modes: Normal / Lecture / Meeting (Reserve2 = 0, 1, 2 respectively) |
| Camera Recording | input_source = AUDIO_SOURCE_MIC | Modes: Normal / Meeting (Reserve2 = 3, 4 respectively) |
| Customization1 | input_source = AUDIO_SOURCE_MIC_AEC | MagiASR need AEC (VR + AEC) |
| Voice Recognition & CTS | input_source = AUDIO_SOURCE_VOICE_RECOGNITION |  |
| Voice Unlock | input_source = AUDIO_SOURCE_VOICE_UNLOCK |  |
| Customization2 | input_source = AUDIO_SOURCE_VOICE_RECOGNITION_AEC | Normal record + AEC |
| Fast Record | According current application | frame_size_ms = 5 ms |

# Appendix: Platform and Development Environments

## Scenario Based DVFS

Figure 7-1. MT6797 DSP Dynamic Voltage Table

We have three DSP clock rate in MT6797, if your application can be run at 110MHz, there is a large power savings for the whole system. We don’t have automatic DVFS to detect current DSP loading and adjust the voltage automatically. What we do is to register a frequency requirement table for a specified application. When the application starts, a provided API should be called to notify the system. Besides, it also needs to call another API to notify the system when it is stopped. The system will adjust the voltage according to the sum of clock rate requirement for active applications.

Since the enable and disable of a function is triggered from the application processor, the judgement of what clock rate is more suitable for current scenario is done by the application processor.

### Implementation

- Registration

kernel-3.18/drivers/misc/mediatek/scp/[project]/scp_helper.h.

kernel-3.18/drivers/misc/mediatek/scp/[project]/scp_helper.c

typedef enum { VOW_FEATURE_ID= 0, OPEN_DSP_FEATURE_ID, SENS_FEATURE_ID, MP3_FEATURE_ID, FLP_FEATURE_ID, RTOS_FEATURE_ID, NUM_FEATURE_ID, } feature_id_t; static scp_feature_table_t feature_table[] = { { .feature = VOW_FEATURE_ID, .freq = 80, .enable = 0, }, { .feature = OPEN_DSP_FEATURE_ID, .freq = 270, .enable = 0, }, { .feature = SENS_FEATURE_ID, .freq = 84, .enable = 0, }, ……. ……. };

First, the feature ID should be added to the feature id table. Then the clock rate requirement of the specified application should also be provided.

- Function Prototype

void register_feature(feature_id_t id); void deregister_feature(feature_id_t id);

When an application is enabled to run, it must call resiger_feature function first to notify the system. The clock rate of the application will be added to the current total clock rate requirement. The system will then adjust the frequency of CPU. When the application is disabled, the deregister_feature should be called so that the system won’t consume unnecessary power.

## DSP Binary (Build Command and Download)

### Configuration

There is an automatic compile dependency check between “PROJECT_CONFIG” of AP and “CFG_PROJECT_CONFIG” of DSP. They should be the same value. If there is a global configuration named “FOO”, and there is a dsp configuration named “CFG_FOO”, only the following three conditions are allowed.

| tinysys_config.h entry | Global ProjectConfig.mk |
| --- | --- |
| #define CFG_FOO | FOO = yes |
| /* CFG_FOO is not set */ | FOO = no |
| #define CFG_FOO VAL | FOO = VAL |

### Build Command

    - Environment Initialzation

      - $ . build/envsetup.sh…………………………………required only once

      - $ lunch full_<PROJECT>-eng………………………re-run this command to switch to other projects.

    - Full Android Build

      - $ mosesq make -j24

    - Module Build for DSP

      - $ mosesq make tinysys-scp -j24

    - Fast Module Build for DSP

      - $ vendor/mediatek/proprietary/tinysys/freertos/source/tools/build_ts.sh

### Built Binary

    - Built DSP Binary Location

      - out\target\product\[project]\obj\TINYSYS_OBJ\tinysys-scp_intermediates\freertos\source\CM4_A\tinysys-scp-CM4_A.bin

      - Check the modified time of the binary if you wanna make sure the binary is updated

    - Update DSP binary by Fastboot

      - $ adb reboot bootloader ………………….. Put the device in fastboot mode

      - $ flashboot.exe scp1 tinysys-scp.bin ………………..Update scp1 from local tinysys-scp.bin

      - $ flashboot.exe scp2 tinysys-scp.bin …………scp2 is only for backup, same with scp1 in MT6797

    - Update DSP binary by MTK Flashtool

      - Flashtool can be used to download Android full load or update DSP binaries only. The following figure shows how to update DSP binaries only. The SCP2 is just a backup of SCP1, they are the same in MT6797.

Figure 7-2. Update DSP Binary by Flashtool

## Code Release

- Get Full Codebase

  - 1. Contact PM and sign an agreement

  - 2. Get a FEX account (contact with the PM)

  - 3. Request the code release

- Special Modem Image Request

  - If you want to use the vendor’s voice call speech enhancement, we need to disable the MTK’s speech enhancement in the Modem Image. Since the modem image is binary released. The customers need to request a modem image with the compile option: “OPEN_DSP_SPEECH_SUPPORT” first.

## Aurisys Framework Class Diagram

Figure 7-3. Aurisys Framework Class Diagram

## Software Patch Note

The following table lists the issues and possible impacts. Please make sure you’ve applied the following patches.

| Feature | ID | Description |
| --- | --- | --- |
| Voice Call | ALPS02562964 | [Impact without patch] Sometimes there is no sound during a voice call [Reproduced ratio] less than 1% |
| Voice Call | ALPS02739463 | [Impact without patch] When the system is very busy (such as a VT call), sometime, it may cause a time out assertion. |
| Record and Aurisys AP side framework | ALPS02695374 | Support Aurisys AP side framework and recording library |

## FAQ

- SCP

  - Abbreviation of System Control Processor. It is because there is some system control tasks are run in this DSP.

- TinySys

  - Our DSP system and peripherals.